����������������������������������������������ͻ

*          External Hard Disk Driver           *

*     (C)opyright ATEN Corp. 1995  Ver 2.xx    *

����������������������������������������������ͼ

Thanks for using one of the best selling external hard disk controllers

EH-100/EH-132.  This advanced external hard disk provides all the

convenience in portability, installation and operation.

There are five files in this cd"\disk2go\EH100" and will be copied to the '\DISK2GO'

directory after SETUP.EXE is executed:

(1) EHD.SYS    : is the device driver and will automatically be appended

to the CONFIG.SYS after SETUP.

(2) EHD.EXE    : executed under DOS prompt to load the device driver.

Execute it if EHD.SYS is not loaded by CONFIG.SYS.

(3) EHD-95.EXE : device driver for Windows95;

to be executed by 'AUTOEXEC.BAT'.

Execute it if EHD.SYS is not loaded by CONFIG.SYS.

(4) SETUP.EXE  : installation program.  Execute it to

install necessary files to a directory called

'DISK2GO' in the bootable disk.

(5) README.DOC : explains the installation procedures.

TABLE OF CONTENTS

Section-1           EH-100/EH-132 Software Installation

Section-2           Software Installation for Advanced Users

Section-3           Hard Disk Write Protection (EH-100 only)

Section-4           Hard Disk Double Space

Section-5           For Low Capacity Hard Disk ( less than 40M)

Section-6           Important Notices

Section-7           Error Messages and Suggestions

Section-8           Troubleshooting

Section-9           Revision Notes

Section-1   EH-100/EH-132 Software Installtion

��������* ������������������������������* ��������

A. Use SETUP.EXE to install the driver routine.

Under the DOS prompt, type the following command:

SETUP DriveName:

"DriveName" is the name of the bootable drive.  For instance, drive C:

is your bootable drive.  Type

SETUP C:

After installation, the screen will display:

EH-100/EH-132 driver routine installation completes.

You must restart the computer to use the portable hard disk.

Now, the EH-100/EH-132 driver routine is installed successfully and

you must restart the computer before using the EH-100/EH-132.

B. To install the second EH-100/EH-132.

The first EH-100/EH-132 driver routine must be installed successfully

before installing the second one.  To install the second EH-100/EH-132

driver routine, type the following command under the DOS prompt:

SETUP DriveName:

This command is the same as the above example.

After installation, the screen will ask:

Install the second EH-100/EH-132 driver routine (Y/N)?

To respond, type:

Y

After installation, the screen will display:

EH-100/EH-132 driver routine installation completes.

You must restart the computer to use the portable hard disk.

Just restart the computer and two EH-100/EH-132 driver routines will

be loaded for operation.

C.  To install the third EH-100/EH-132.

The first two EH-100/EH-132 driver routines must be installed successfully

before installing the third one.  To install the third EH-100/EH-132

driver routine, type the following command under the DOS prompt:

SETUP DriveName:

This command is the same as the above example.

Section-2   Software Installation for Advanced Users

��������* ������������������������������* ��������

������������������������������������������������������������������������*

* Your      * You want to   *   You should     *      Note              *

* system is * use EHD after *                  *                        *

������������������������������������������������������������������������*

*           * computer      * add EHD.SYS to   * device driver loaded   *

*  DOS      * resets        * CONFIG.SYS       * when computer resets   *

*   &       ������������������������������������������������������������*

*  Windows  * running EHD   * run EHD.EXE      * device driver loaded   *

*           * device driver * under DOS prompt * when needed            *

������������������������������������������������������������������������*

*           *               * add EHD.SYS to   *.device driver loaded   *

*           *               * CONFIG.SYS       * when computer resets   *

*           *               *                  *.may slow down Windows95*

* Windows95 * opening       ��������������������������������������������*

*           * Windows95     * add EHD-95.EXE   *.device driver loaded   *

*           *               * to AUTOEXEC.BAT  * when computer resets   *

*           *               *                  *.you may not use FDISK  *

*           *               *                  *.does not slow down     *

*           *               *                  * Windows95.             *

������������������������������������������������������������������������*

A. For occasional EHD users under DOS or Windows:

To save system memory, you may remark the 'EHD.SYS' in the 'CONFIG.SYS',

and execute 'EHD.EXE' under DOS when EHD is needed.

Also, specify a higher drive for 'LASTDRIVE' in the 'CONFIG.SYS'.

for example:

add 'REM ' in front of 'device=C:\DISK2GO\EHD.SYS' in 'CONFIG.SYS'.

then, under DOS prompt, execute

C:>EHD <Enter>

B. For Windows95 user who wants to use FDISK.EXE, please follow these steps:

(However, only one EH-100/EH-132 can be connected to a computer. )

1. Before executing 'FDISK.EXE' run 'C:\DISK2GO\SETUP /FDISK'.

2. Restart your computer.

3. Run 'FDISK.EXE'.

4. Run 'C:\DISK2GO\SETUP /95' to speed up the Windows95 operation.

C. To unload the EHD resident program from the system memory under DOS prompt,

execute

C:>EHD /U <Enter>

Section-3    Hard Disk Write Protection (EH-100 only)

��������* ������������������������������* ��������

.If your computer loads SMARTDRV by AUTOEXEC.BAT, the write-back

function of the SMARTDRV must be disabled for all EH-100's external

logical drives.  Otherwise, the computer hangs when trying to write

data to a write-protect external hard disk on EH-100.

For instance, to disable SMARTDRV write-back function on logical

drives D:, E:, and F:, change the SMARTDRV command in the AUTOEXEC.BAT

to:

SMARTDRV D E F

Section-4     Hard Disk Double Space

��������* ������������������������������* ��������

.Your PC's hard disk is not double-spaced, and you try to read a double-

spaced hard disk X: on EH-100/EH-132.  First of all, you must double-

space one of your PC's hard disks.  Second of all, enter the following

command ( an MS-DOS 6.0 command ) after starting your computer before

accessing the external hard disk X:.

C:\DBLSPACE /MOUNT X:

Section-5   For Low Capacity Hard Disk ( less than 40M)

��������* ������������������������������* ��������

. You have to setup the hard disk parameters (cylinder, sectors, heads)

if you have Error-05 during installation.

. Please modify your CONFIG.SYS, for example:

device =X:\DISK2GO\EHD.SYS /t:n   (n: 1~46, disk type number)

. This option is invalid for PS/2, because its BIOS does not provide

hard disk parameter tables.

. Do not use more than one external hard disk when this option is used.

Section-6     Important Notice

��������* ������������������������������* ��������

1. EH-100/EH-132 hard disk controller uses all standard Centronics signals

for data transfer; therefore, should trouble occurred during installation

and operation, please check the LPT port signal pins.

2. If you manually modify the CONFIG.SYS by any word processor program,

A. All the EHD.SYS commands in the CONFIG.SYS file must follow each other.

B. The EHD.SYS commands must be place BEFORE the DBLSPACE.SYS which is

the double-space function provided by DOS 6.0 and up.

3. Use only DC 5V for EH-100's power jack.

4. Use EH-100/EH-132 for computers with 80286 CPU and up.

5. For computers with two EH-100/EH-132s, if the first LPT port fails

the driver routine will display error messages from ERROR-2 to ERROR-6 twice.

When the errors show, please take out or replace the first LPT port card

and try again.

6. For DOS version under 5.0, FDISK.EXE can only control up to two hard disks.

Section-7     Error Messages and Suggestion

��������* ������������������������������* ��������

.When you encounter an error while operating the EH-100/EH-132, follow these

steps to correct the problem.

Error Code & Causes                       Suggestion

������������������������������������* ��������������������������������������*

. Error-01: EH-100/EH-132 not Found.

a) Power adapter or DB-25 cable is   . Check if EH-100/EH-132 is securely

not well connected.                  connected to a LPT port.

b) EH-100/EH-132 not found, The LPT  . Replace the LPT port card.

port does not provide enough

signals for the EH-100/EH-132 to

work with.

������������������������������������* ��������������������������������������*

. Error-02: Data transfer mode error.

a) Faulty DB-25 cable.               . Replace or reconnect the DB-25 cable

to the EH-100/EH-132.

b) The LPT port does not provide     . Append the '/N' option to the

complete byte mode operations,      CONFIG.SYS and force the EH-100/EH-132

the EH-100/EH-132 may use           to work in nibble node, e.g.,

nibble mode only.                       device =X:\EHD.SYS /N

reset the computer and try again.

������������������������������������* ��������������������������������������*

. Error-03: Incompatible LPT port error.

a) The LPT port of the first EH-100/  . Swap LPT port cards or replace

EH-132 controller supports           LPT port cards with the same one

different data transfer mode from    as the first card.

the others.

������������������������������������* ��������������������������������������*

. Error-04: External Hard Disk does not

work.

a) The hard disk is not well         . Please check for secured connection.

connected.

b) The 44-pin IDE cable is not       . Make sure the IDE cable from the PCB

well connected.                     of EH-100/EH-132 is well connected

to the hard disk.

c) The hard disk is incorrectly set  . Check the hard disk user manual and

to master or slave mode by its      configure the hard disk to a proper

jumper.                             mode.

������������������������������������* ��������������������������������������*

. Error-05: Driver: X Hard Disk

parameter not found.

a) Faulty DB-25 cable.               . Please connect the EH-100/EH-132 to

your PC by the factory-supplied DB-25

female-to-male cable.  Or, Choose

a high quality cable shorter than

one meter for better noise immunity.

b) Your hard disk does not provide  . Append the '/t' option to the

0ECh command for reading hard      CONFIG.SYS with appropriate disk type

disk parameter.                    number, e.g.

device =X:\EHD.SYS /t:n

The disk type number (n:1~46) can be

found in your hard disk user's manual.

c) The LPT port does not provide     . Replace the LPT port card.

enough signals for the EH-100/

EH-132 to work with.

������������������������������������* ��������������������������������������*

. Error-06: Please use DOS 3.3 or

higher for EH-100/EH-132

driver routine.

a) EH-100/EH-132 driver supports     . Replace your DOS system.

DOS 3.3 and later.

������������������������������������* ��������������������������������������*

. Error-07: Can not reset the hard

disk after reading its

parameters.

a) Your hard disk parameters can     . Append the '/t' option to the

not be initialized.                 CONFIG.SYS with appropriate disk type

number, e.g.

device =X:\DISK2GO\EHD.SYS /t:n

The disk type number (n:1~46) can be

found in your hard disk user's manual.

������������������������������������* ��������������������������������������*

. Error-08: Can not write or read the

EH-100/EH-132 via printer

port correctly.

a) Your printer port may be too      . Slow down the driver by appending

slow to match the speed of the      the '/s' option to your CONFIG.SYS,

EH-100/EH-132 driver.               e.g.

device =X:\DISK2GO\EHD.SYS /s

b) Your printer port card is too     . Replace the LPT port card.

slow to operate.

������������������������������������* ��������������������������������������*

. Warning!! Only the lower 32M Bytes

are accessible for

DOS 3.3.                  . External Hard disk is formatted by

DOS 3.3 later, but the boot DOS

is Ver 3.3. So, the maximum

accessible disk space is 32M bytes.

������������������������������������* ��������������������������������������*

. Warning!! Maximum mountable logical

disks is 26.

. Total number of logical drives exceeds

26. Any drive higher than 26 would

not be accessible by the DOS.

Please repartition & format the disk.

������������������������������������* ��������������������������������������*

. (By DOS 5.0 & upper)

Invalid media type reading drive X  . Logical drive 'X' is not formatted.

Abort, Retry, Fail?                   Please type 'F'  then run FORMAT.COM

to format the logical drive.

. (By DOS 3.3)

Non-DOS disk error reading drive X

Abort, Retry, Fail?

������������������������������������* ��������������������������������������*

. (By DOS)

Write protect error writing drive X

Abort, Retry, Fail?

a) Write protect function is         . To disable write protect, slide

enabled, data can not be written    the write-protect switch on EH-100 to

to the external hard disk 'X'.      close position and type 'R' to Retry.

������������������������������������* ��������������������������������������*

Section-8         Troubleshooting

��������* ������������������������������* ��������

Symptom                             Suggestion

������������������������������������* ��������������������������������������*

1) The power LED indicator is off      . Check keyboard power adapter or

when power is applied.                external AC power for secured

connection.

������������������������������������* ��������������������������������������*

2) No EH-100/EH-132 message after      . Make sure DEVICE=EHD.SYS command

computer resets.                      is added to the 'CONFIG.SYS' file.

. Make sure the 'PATH' definition

has the directory of DISK2GO driver.

������������������������������������* ��������������������������������������*

3) The computer does not pause for     . Add the '/P' option after the command

an error when it loads EH-100/        Device= EHD.SYS in the CONFIG.SYS

EH-132 driver.                        file.

������������������������������������* ��������������������������������������*

4) FDISK fails to work properly.       . The hard disk is protected by the

protection switch.  Disable the

protection by sliding the switch.

������������������������������������* ��������������������������������������*

5) At some rare occasions, a formatted . The hard disk can not provide its

hard disk capacity becomes much       parameter to the EH-100/EH-132 driver,

less than specified by the disk       add the '/T' option after the

manufactures.  And the hard disk      Device= EHD.SYS in your CONFIG.SYS.

is  unable to read nor write.         (Check manual section 2.3 for detail.)

������������������������������������* ��������������������������������������*

6) Both LPT ports are connected with   . EH-100/EH-132 driver always tries to

EH-100/EH-132s.  An installation      install itself to the first LPT port

error occurs while installing the     available. That's why the same error

first EH-100/EH-132, and the same     on the same LPT port could be found

error occurs on the same LPT while    twice.

installing the second EH-100 /      . For error #01 ~ #03, replace the

EH-132 drive.                         first LPT port.

. For error #04, replace the EH-100

on the first LPT port.

������������������������������������* ��������������������������������������*

7) Can not load CD-ROM device under    . Run 'C:\DISK2GO\SETUP /95'then

Windows95.                            restart your computer .

������������������������������������* ��������������������������������������*

#